CSE 4207 CT 4 Assignment

Roll No: 1903178

Assignment Problem: 7-bit ALU operation with NOT and Shift Right operations.

Category: C

Word Size = 7

ALU Operations = NOT and Shift Right

Solution:

Video:

| Have you uploaded the video? | YES |
| --- | --- |
| Check List 1: Have you explained the design using testbenches to prove that your circuit is working correctly and giving correct results? | YES |
| Check List 2: Have you shown full synthesis results showing all the required info including RTL Synthesis, RTL Floorplan, RTL Power Analysis, GDS and Heatmap (for details, see assignment template doc)? | YES |
| NB: Failing to upload video will cause heavy point penalty (5-6 Marks) |  |
| NB: Failing to add any required info will cause point penalty (1-2 Marks) |  |

HDL Code:

| Check List: Have you added all the modules written in verilog including ALU, ALU_OP1, ALU_OP2, CONTROLLER, TOP, TOP_TESTBENCH, ALU_TESTBENCH, CONTROLLER_TESTBENCH? | YES |
| --- | --- |
| NB: Failing to add any required info will cause point penalty (1-2 Marks) |  |

ALU_7bit.v

module ALU_7bit ( input wire [6:0] A, B, input wire OP, // 0: NOT, 1: SHR output reg [6:0] result, output wire ZF // Zero flag ); wire [6:0] R_NOT, R_SHR; // Instantiate modules ALU_NOT_7bit NOT1 (.A(A), .result(R_NOT)); ALU_SHR_7bit SHR1 (.in(A), .shift(B[2:0]), .out(R_SHR)); always @(*) begin case (OP) 1'b0: result = R_NOT; // NOT operation 1'b1: result = R_SHR; // SHR operation default: result = 7'b0000000; endcase end // Zero flag (1 when result is all zeros) assign ZF = (result == 7'b0000000); endmodule

ALU_7bit_tb.v

`timescale 1ns/1ns module ALU_7bit_tb; reg [6:0] A, B; reg OP; wire [6:0] result; wire ZF; ALU_7bit uut ( .A(A), .B(B), .OP(OP), .result(result), .ZF(ZF) ); initial begin $dumpfile("alu_7bit_test.vcd"); $dumpvars(0, ALU_7bit_tb); // Test NOT operation A = 7'b1010101; B = 7'b0000000; OP = 1'b0; #10; // Test SHR operation with different shift amounts A = 7'b1100110; B = 7'b0000001; OP = 1'b1; // Shift by 1 #10; A = 7'b1100110; B = 7'b0000011; OP = 1'b1; // Shift by 3 #10; A = 7'b1100110; B = 7'b0000111; OP = 1'b1; // Shift by 7 (all bits out) #10; // Test zero flag A = 7'b0000000; B = 7'b0000000; OP = 1'b0; // NOT of 0 should be all 1s #10; A = 7'b0000001; B = 7'b0000001; OP = 1'b1; // Shift 1 right by 1 (should be 0) #10; $finish; end initial begin $monitor("Time=%3t A=%b B=%b OP=%b -> Result=%b ZF=%b", $time, A, B, OP, result, ZF); end endmodule

ALU_NOT_7bit.v

module ALU_NOT_7bit ( input wire [6:0] A, output wire [6:0] result ); assign result = ~A; endmodule

ALU_SHR_7bit.v

module ALU_SHR_7bit ( input wire [6:0] in, input wire [2:0] shift, // Shift amount (0-7) output reg [6:0] out ); always @(*) begin case(shift) 3'b000: out = in; 3'b001: out = {1'b0, in[6:1]}; 3'b010: out = {2'b00, in[6:2]}; 3'b011: out = {3'b000, in[6:3]}; 3'b100: out = {4'b0000, in[6:4]}; 3'b101: out = {5'b00000, in[6:5]}; 3'b110: out = {6'b000000, in[6]}; 3'b111: out = 7'b0000000; // Shift all bits out endcase end endmodule

top.v

module top ( input wire clk, reset, output wire [6:0] result, output wire flag_gt_zero ); wire [6:0] A, B; wire OP; wire [6:0] alu_result; wire ZF; controller fsm ( .clk(clk), .reset(reset), .A(A), .B(B), .OP(OP) ); ALU_7bit datapath ( .A(A), .B(B), .OP(OP), .result(alu_result), .ZF(ZF) ); assign result = alu_result; assign flag_gt_zero = (alu_result != 7'b0); // Direct comparison endmodule

top_tb.v

`timescale 1ns/1ns module top_tb; reg clk, reset; wire [6:0] result; wire flag_gt_zero; top uut ( .clk(clk), .reset(reset), .result(result), .flag_gt_zero(flag_gt_zero) ); initial begin clk = 0; forever #5 clk = ~clk; end initial begin $dumpfile("top_test.vcd"); $dumpvars(0, top_tb); reset = 1; #10; reset = 0; #150; // Cover all states $finish; end initial begin $monitor("Time=%0t Result=%b Flag=%b", $time, result, flag_gt_zero); end endmodule

controller.v

module controller ( input wire clk, reset, output reg [6:0] A, B, output reg OP ); reg [2:0] pstate, nstate; parameter START = 3'b000, ONE = 3'b001, TWO = 3'b010, THREE = 3'b011, FINISH = 3'b100; // State register always @(posedge clk or posedge reset) begin if (reset) pstate <= START; else pstate <= nstate; end // Next state and output logic always @(*) begin A = 7'b0; B = 7'b0; OP = 1'b0; nstate = pstate; case (pstate) START: nstate = ONE; ONE: begin // Test NOT operation (result non-zero) A = 7'b1010101; OP = 1'b0; // NOT opcode nstate = TWO; end TWO: begin // Test SHR operation (result non-zero) A = 7'b1100110; B = 7'b0000001; // Shift by 1 OP = 1'b1; // SHR opcode nstate = THREE; end THREE: begin // Test SHR operation (force result=0) A = 7'b0000001; B = 7'b0000111; // Shift by 7 (all bits out) OP = 1'b1; nstate = FINISH; end FINISH: nstate = START; default: nstate = START; endcase end endmodule

controller_tb.v

`timescale 1ns/1ns module controller_tb; reg clk, reset; wire [6:0] A, B; wire OP; controller uut ( .clk(clk), .reset(reset), .A(A), .B(B), .OP(OP) ); initial begin clk = 0; forever #5 clk = ~clk; end initial begin $dumpfile("controller_test.vcd"); $dumpvars(0, controller_tb); reset = 1; #10; reset = 0; #100; $finish; end initial begin $monitor("Time=%3t State=%b A=%b B=%b OP=%b", $time, uut.pstate, A, B, OP); end endmodule

RTL Timing Diagram:

| Check List: Have you added all the timing diagrams of ALU_TESTBENCH, CONTROLLER_TESTBENCH, TOP_TESTBENCH? | YES |
| --- | --- |
| NB: Failing to add any required info will cause point penalty (1-2 Marks) |  |

top_tb.v

controller_tb.v

ALU_7bit_tb.v

RTL Synthesis (130nm Skywater PDK with OpenLane toolchain):

| Check List: Have you added RTL synthesis summary, RTL synthesized design figure and Standard cell usage in synthesized design? | YES |
| --- | --- |
| NB: Failing to add any required info will cause point penalty (1-2 Marks) |  |

RTL synthesis summary

RTL synthesis summary (TOP)

RTL synthesis summary (ALU)

RTL synthesized design figure:

Hierarchy (top)

Primitive techmap (top)

Hierarchy (ALU)

Primitive techmap (ALU)

Standard cell usage in synthesized design

Standard Cells(Top)

Standard Cells(ALU)

RTL Floorplan (130nm Skywater PDK with OpenLane toolchain)

| Check List: Have you added RTL Floorplan info? | YES |
| --- | --- |
| NB: Failing to add any required info will cause point penalty (1-2 Marks) |  |

RTL Floorplan (ALU): or_metrics_out.json file info is given here

RTL Floorplan (top): or_metrics_out.json file info is given here

RTL Power Analysis (130nm Skywater PDK with OpenLane toolchain)

| Check List: Have you added RTL Power Analysis info? | YES |
| --- | --- |
| NB: Failing to add any required info will cause point penalty (1-2 Marks) |  |

RTL Power Analysis (ALU)

RTL Power Analysis (top)

GDS Layout (130nm Skywater PDK with OpenLane toolchain)

| Check List: Have you added the GDS Layout figure? | YES |
| --- | --- |
| NB: Failing to add any required info will cause point penalty (1-2 Marks) |  |

GDS Layout (ALU)

GDS Layout (top)

Heatmap (130nm Skywater PDK with OpenLane toolchain)

| Check List: Have you added the heatmap? | YES/NO |
| --- | --- |
| NB: Failing to add any required info will cause point penalty (1-2 Marks) |  |

(Following figure is showing what info need to be shown /Just copy paste these figures)

Heatmap (ALU)

Heatmap (top)